Full-spectrum LED (Light Emitting Diode) Product

  - GENERAL

- The product shall be a Fiilex P5F manufactured by DiCon Fiberoptics Inc.

- The product shall be a high-intensity LED illuminator utilizing a Dense Matrix 3D LED Array System comprised of at least 7 different LED chip colors

- The product shall comply with the USITT DMX-512 standard.

- The product shall comply with the current PLASA ANSI E1.20- 2010 remote device management (RDM) standard.

- Fiilex, a DiCon FiberOptics Inc. brand, shall provide all LED products to ensure color consistency.

B.    Each LED fixture shall be tested and optimized for optimal photometric performance.

  - PHYSICAL

- The dimensions of the fixture shall be 14.2” W x 11.8” H x 13.0” D (36.0 x 30.0 x 32.9 cm) and weigh approximately 9.6 lbs (4.4 kg). The following shall be provided:

  - Fiilex P5F fixture, consisting of

- P5F COLOR Head & Yoke Unit

- 175W, 48V, Clamp Mount PSU

- P5 PSU AC Jumper Cable 5ft

- P5F Barndoor 8 leaf (FLXP5F-BD8-BK/FLXP5F-BD8-WH)

- The housing shall have a black or white finish.

- The housing material shall be PC (polycarbonate) + ABS (acrylonitrile-butadiene-styrene).

- Mounting Type shall have Track or Clamp selection

- Product Cooling and Electronics will be internal systems.

  - ENVIRONMENTAL AND AGENCY COMPLIANCE

- The product shall comply with UKCA marking requirements per BS EN IEC 60598-1 and BS EN IEC 60598-2-1.

- The product shall meet FCC Part 15 Subpart B for EMI compliance.

- The product shall be ETL certified to UL 1574 and CSA C22.2 for track lighting systems.

- The product shall meet CE LVD standards per BS EN IEC 60598-1 and BS EN IEC 60598-2-1.

- The product shall meet CE photobiological Safety standard EN 62471.

- The product shall meet CE EMC standards, including EN IEC 55015, EN IEC 61000-3-2, EN 61000-3-3, and EN 61547.

- The product shall also comply with UK EMC standards equivalent to CE, including BS EN IEC 55015, BS EN IEC 61000-3-2, BS EN IEC 61000-3-3, and BS EN 61547.

- The fixture shall comply with RoHS (Restriction of Hazardous Substances) and TAA (Trade Agreements Act) regulations.

- The product shall be rated for IP-25 and able to sustain operation at full intensity while actively being sprayed with water from all directions.

  - Thermal

- Product heat management shall be achieved through forced cooling.

- The cooling fans shall be rated for a minimum operational lifespan of 60,000 hours.

- The product shall utilize advanced thermal management systems to maintain LED life to an average of 70% intensity after 50,000 hours of use.

- The product shall operate in an ambient temperature range of 32°F (0° C) minimum to 104°F (40° C) maximum.

  - Electrical

- The product shall have an auto-ranging 100 V to 240 V 50/60 Hz power supply unit.

- The product shall have a maximum draw of 175W.

- DC input Voltage shall be 24- 48V.

- The product requires power from a non-dimming source.

- Products shall have dynamic thermal monitoring at multiple locations in the LED array, control board, and other electronics to prevent thermal shift of color or intensity.

- Product power input shall have current-limiting fuse protection.

- The power supply shall have power factor correction.

  - OPTICAL DATA

- The product shall contain a patented Dense Matrix LED Light Source manufactured by DiCon FiberOptics, Inc.

- The fixture shall have a 12-55 degree native beam angle, with user-adjustable and lockable barn doors to cut the beam.

- All LEDs used in the product shall be manufactured by DiCon FiberOptics, ensuring high brightness and proven quality.

- DiCon FiberOptics, Inc. shall utilize an advanced production LED binning process to maintain color consistency.

- LED emitters should be rated for a nominal 50,000-hour LED life to 70% intensity.

- All LED products (100% of each lot) shall undergo a minimum three-hour burn-in test during manufacturing.

- The LED system shall comply with all relevant patents.

  - Color

- The product shall utilize a Dense Array of 92 LED emitters enclosed under a single half-ball lens with a diameter of 22mm.

- The Color Rendering Index (CRI) shall be 95 at 3200K, 96 at 5600K, and 97 at 6500K.

- The Television Lighting Consistency Index (TLCI) shall be 90 at 3200K, 95 at 5600K, and 96 at 6500K.

- The Correlated Color Temperature (CCT) shall be dynamic, ranging between 2000° K and 10000° K.

- The Hue range will be adjustable ±1.0 Magenta Plus/Green.

  - Calibration

- Every LED Fixture should undergo a stringent calibration process (500+ points) to ensure color consistency between fixtures and accuracy across the entire CCT range and color space.

  - Intensity Measurements

- The fixture must produce

  - 12° beam: Narrow focus with approximately 35,000 cd peak intensity and rapid falloff.

  - 55° beam: Wide coverage with approximately 4,100 cd peak intensity and even distribution.

- Focus

  - Color output @3200k distance 3’ (0.9m) at 3150fc/33907lux, distance 10’ (3.0m) at 359fc/3865lux,  distance 15’ (4.6m) at 128fc/1378lux.

  - Color output @5600k distance 3’ (0.9m) at 3296fc/35478lux, distance 10’ (3.0m) at 376fc/4048lux,  distance 15’ (4.6m) at 134fc/1443lux.

  - At 3 feet (0.9 meters), the beam spreads to 0.7 feet (0.2 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 2.1 feet (0.6 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 3.5 feet (1.1 meters) in diameter.

- Flood

  - Color output @3200k distance 3’ (0.9m) at 385fc/4145lux, distance 10’ (3.0m) at 45fc/485lux,  distance 15’ (4.6m) at 17fc/183lux.

  - Color output @5600k distance 3’ (0.9m) at 406fc/4371lux, distance 10’ (3.0m) at 47fc/506lux,  distance 15’ (4.6m) at 18fc/194lux.

  - At 3 feet (0.9 meters), the beam spreads to 0.9 feet (0.6 meters) in diameter.

  - At 10 feet (3.0 meters), the beam spreads to 8.6 feet (2.6 meters) in diameter.

  - At 15 feet (4.6 meters), the beam spreads to 14.4 feet (4.4 meters) in diameter.

  - Dimming

- The LED system will use an analog dimming system for a flicker-free output, tested with cameras over 20,000 fps.

- The LED system shall use 16-bit nonlinear scaling techniques for high-resolution dimming.

- The product shall have a selectable dimming curve to simulate incandescent dimming curves consistent with the following:

  - Linear

  - Gamma

- The dimming curve shall be optimized for smooth dimming over longer timed fades.

- The product shall have a selectable dimming mode consistent with the following:

  - Smooth

  - Sharp

  - REQUIRED FEATURE SET

- The product shall offer 6 DMX Control Mode Options.

- Product Modes will include: CLR (All 26 Modes), EZ (All 13  EZ Modes), RGB Mode (RGB Mode Only), CCT (Tunable CCT Mode Only), 3200 (Fixed 3200K), and 5600 (Fixed 5600K).

- The product shall offer 4 Fan modes: Auto, Full Speed, Off, and Silent

- The product shall offer a noise level not to exceed 23 dBa.

- The product shall have an over-temperature protection that disables the fixture above 102 Celsius, as measured by a thermistor embedded in the LED Array.

- The product shall offer a 631 (BTUs/hour) Max.

- The product shall offer a ±1.0 Magenta/Green shift via DMX and onboard control.

- The product shall offer full RGB color over the entire CIE 1931 color space.

- The product shall offer user-selectable Color Temperature settings.

- The product shall offer user-selectable dimming settings.

- The product shall contain a direct power/data connection.

- The product shall contain two manual knobs on the back of the fixture to control all fixture parameters.

- The product yoke assembly shall include two additional mounting point options, allowing for a lower-profile installation configuration.

- All provided products will contain the above feature set.

  - Control and User Interface

- The product shall be USITT DMX 512A-compatible via in and through 5-Pin XLR DMX Connectors or TRRS Connectors.

- The product shall offer control via DMX and RDM.

- The product shall be compatible with the ANSI RDM E1.20 standard.

- Product mode, DMX address, and fixture info shall be accessible via RDM protocol for modification from a suitably equipped control console or RDM controller.

- Temperature sensors within the luminaire shall be viewable on the product control panel in real time.

- All offered products will come with the listed RDM Compatibility.

- The product shall be equipped with an OLED display.

- The product shall be equipped with a 3 Knob user interface.

- The product shall offer stand-alone functionality, eliminating the need for a console.

- The product shall ship with full RGB capability.

- Products without stand-alone operation features described above shall not be acceptable.

-END-